                                   MATERIAL SAFETY DATA SHEET



SECTION 1. PRODUCT AND COMPANY IDENTIFICATION
MUSCA DOOM ATTRACTANT GEL

PRODUCT CODE: PCO-MUSCA DOOM, REFERENCE #: 57117
PRODUCT NAME: MUSCA DOOM ATTRACTANT GEL CHEMICAL FAMILY: Compound

	MANUFACTURER NAME AND ADDRESS:
	
	TELEPHONE NUMBERS:
	DATES:

	Farnam Companies, Inc.
	24 hour emergency number
	(602)285-1660
	Date Created: 03/24/1992

	301 West Osborn Road
	Business hours
	(602)285-1660
	Revision: 04/26/2001

	Phoenix, AZ. 85013
	Marketing
	(602)285-1660
	Printed: 01/12/2005




SECTION 2. COMPOSITION/INFORMATION ON INGREDIENTS
MUSCA DOOM ATTRACTANT GEL

	  
	HAZARDOUS COMPONENTS (CHEMICAL NAME)  
	CAS # 
	OSHA PEL  
	ACGIH TLV  
	OTHER LIMITS   
	PERCENTAGE   
	RTECS #

	1. 
	Ethyl Alchol 
	64-17-5   
	1000 ppm   
	  
	
	31.59 % 
	KQ6300000

	2. 
	Isopropyl alcohol {sec-Propyl alcohol; IPA; 2-Propanol} 
	67-63-0   
	400 ppm   
	  
	
	1.66 % 
	NT8050000




SECTION 3. HAZARDS IDENTIFICATION
MUSCA DOOM ATTRACTANT GEL

EMERGENCY OVERVIEW 

ROUTE(S) OF ENTRY: Inhalation? Yes , Skin? Yes , Eyes? Yes , Ingestion? Yes
POTENTIAL HEALTH EFFECTS (ACUTE AND CHRONIC) 

CARCINOGENITICY: NTP? No , IARC Monographs? No , OSHA Regulated? No
CARCINOGENICITY/OTHER INFORMATION 

RECOMMENDED EXPOSURE LIMITS: NE LD 50/LC 50: 
SIGNS AND SYMPTOMS OF EXPOSURE 
Eye irritation, mucous membrane and upper respiratory tract irritation. Breathing vapors may cause drowsiness. 

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE 
NE 



SECTION 4. FIRST AID MEASURES
MUSCA DOOM ATTRACTANT GEL

EMERGENCY AND FIRST AID PROCEDURES 
IF SWALLOWED: Do NOT induce vomiting. Drink two glasses of water and contact physician or Poison Control Center. Never give fluids or induce vomiting if patient is unconscious or having convulsions. 
IF INHALED: Remove to fresh air, administer artificial respiration if required. Obtain medical attention. 
IF IN EYES: Flush eyes with water for 15 minutes; obtain medical attention. 
IF ON SKIN: Wash with soap and water; obtain medical attention if irritation persists. 

NOTE TO PHYSICIAN 



SECTION 5. FIRE FIGHTING MEASURES
MUSCA DOOM ATTRACTANT GEL

FLASH PT: 66.00 F METHOD USED: Pensky-Martin 
EXPLOSIVE LIMITS: LEL: 3% UEL: 19% 
EXTINGUISHING MEDIA 
CO2, water, foam, or dry chemical 

FIRE FIGHTING INSTRUCTIONS 
Approved respirators should be worn. Use water spray to cool equipment, containers and to disperse vapors. 

FLAMMABLE PROPERTIES AND HAZARDS 
Flames (from alcohol carrier) may be difficult to see because they are virtually colorless. 

HAZARDOUS COMBUSTION PRODUCTS 



SECTION 6. ACCIDENTAL RELEASE MEASURES
MUSCA DOOM ATTRACTANT GEL

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED 
Ventilate area and eliminate all ignition sources. Large spills: dike and remove into drums. Small spills: wipe up or cover with absorbent and remove to container. Wash contaminated area with soap and water followed by an alcohol rinse, taking care to mop up all rinsate (do not spread). CAUTION: Alcohol is flammable. 



SECTION 7. HANDLING AND STORAGE
MUSCA DOOM ATTRACTANT GEL

HAZARD LABEL INFORMATION: 

	Avoid contact w/eyes/skin/clothing  
	


PRECAUTIONS TO BE TAKEN IN HANDLING 
Do not use/store near heat, sparks, and open flame. Avoid inhalation of vapors. Store in a cool dry location away from food or feed. 

PRECAUTIONS TO BE TAKEN IN STORING 

OTHER PRECAUTIONS 
KEEP OUT OF REACH OF CHILDREN 



SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION
MUSCA DOOM ATTRACTANT GEL

RESPIRATORY EQUIPMENT (SPECIFY TYPE) 
OSHA/MSHA approved respirator if P.E.L.s exceeded 

EYE PROTECTION 
chemical goggles 

PROTECTIVE GLOVES 
Latex, PVC or neoprene 

OTHER PROTECTIVE CLOTHING 
rubber boots, apron if exposure is likely 

ENGINEERING CONTROLS (VENTILATION ETC.) 
Local Exhaust: sufficient 
Special: 
Mechanical (Gen): 
Other: 

WORK/HYGIENIC/MAINTENANCE PRACTICES 
Wash hands before eating, smoking or using restroom. 

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES
MUSCA DOOM ATTRACTANT GEL

	PHYSICAL STATES: 
	[ ] Gas , [ X ] Liquid , [ ] Solid

	BOILING POINT: 
	N.A.

	MELTING POINT:  
	N.A.

	SPECIFIC GRAVITY (WATER = 1): 
	0.95 at 25.0 C

	VAPOR PRESSURE (VS. AIR OR MM HG): 
	

	VAPOR DENSITY (VS. AIR = 1): 
	

	EVAPORATION RATE (VS BUTYL ACETATE=1):  
	>1N.A.

	SOLUBILITY IN WATER: 
	>96%

	OTHER SOLUBILITY NOTES: 
	Soluble

	PERCENT VOLATILE: 
	83.000 % by volume.

	PH: 
	4.5+-0.05


APPEARANCE AND ODOR 
Translucent white to yellowish viscous liquid with strong fishy-alcohol odor 



SECTION 10. STABILITY AND REACTIVITY
MUSCA DOOM ATTRACTANT GEL

STABILITY: Unstable [ ] Stable [ X ]
CONDITIONS TO AVOID - INSTABILITY 
heat, sparks, flames and other ignition sources 

INCOMPATIBILITY - MATERIALS TO AVOID 
Acids, acid chlorides, acid anhydrides, oxidizing agents, heat, sparks, flame, and other ignition sources. 

HAZARDOUS DECOMPOSITION OR BYPRODUCTS 
HCl, CO, CO2, and N compounds 

HAZARDOUS POLYMERIZATION: Will occur [ ] Will not occur [ X ]
CONDITIONS TO AVOID - HAZARDOUS POLYMERIZATION 
will not occur 



SECTION 11. TOXICOLOGICAL INFORMATION
MUSCA DOOM ATTRACTANT GEL



SECTION 12. ECOLOGICAL INFORMATION
MUSCA DOOM ATTRACTANT GEL



SECTION 13. DISPOSAL CONSIDERATIONS
MUSCA DOOM ATTRACTANT GEL

WASTE DISPOSAL METHOD 
Dispose of in accordance with applicable local, State and Federal regulations. 



SECTION 14. TRANSPORT INFORMATION
MUSCA DOOM ATTRACTANT GEL

DOT PROPER SHIPPING NAME 


DOT HAZARD LABEL: Consumer Commodity, ORM-D 
UN/NA NUMBER: 
ADDITIONAL TRANSPORT INFORMATION 



SECTION 15. REGULATORY INFORMATION
MUSCA DOOM ATTRACTANT GEL



SECTION 16. OTHER INFORMATION
MUSCA DOOM ATTRACTANT GEL


The information and data herein are believed to be accurate and have been compiled from sources believed to be reliable. It is offered for your consideration, investigation and verification.
